WF8630L Bi-directional Indoor User Amplifier

- SUMMARY

WF8630L is specialized for construction of the network.. High compact circuit and Aluminium-alloy, drawbench of surface. good index and fine outfit.

- PERFORMANCE CHARACTERISTICS

  - Adopt SOT-115 encapsulation RF module on access, reliable non-liner. Choose  low-noise micro-wave push-pull circuit, small distortion and high S/N ratio.

  - Set up EQ and ATT on the right place, more convenient of debugging.

  - Small power consumption, high reliable, good performance and price ratio.

- PRINCIPLE DRAWING

- CONSTRUCTION EXPLANATION

5. Performance Parameters

| Item | Unit | parameters |  |
| --- | --- | --- | --- |
| Forward Transmission |  |  |  |
| Frequency Range | MHz |  | (47) 85~862 |
| Rated Gain | dB |  | 30 |
| Min Fill Gain | dB |  | ≥30 |
| Rated Input Level | dBμV |  | 70±2 |
| Rated Output Level | dBμV |  | 100 |
| Flatness in Band | dB |  | ±1 |
| Noise Figure | dB |  | ≤10 |
| Return Loss | dB |  | ≥14 |
| C/CTB (84 PAL-D) | dB |  | ≥58 |
| C/CSO (84PAL-D) | dB |  | ≥56 |
| Signal to Alternative Noise Ratio | dB |  | ≤2% |
| Gain Stability | dB |  | ≤± 1.0 |
| Voltage Stroke | KV |  | 5 |
| Backward Transmission |  |  |  |
| Frequency Range | MHz | 5~(30)65(or specified by user) |  |
| Rated Gain | dB | ≥15 |  |
| Flatness In Band | dB | ±1 |  |
| Noise Figure | dB | ≤12 |  |
| Return Loss | dB | ≥16 |  |
| MAX Output Level | dBμV | ≥110 |  |
| Carrier to second order inter modulation ratio | dB | ≥52 |  |
| Carrier to Alternative noise ratio | 2% | ＜2 |  |
| General Response |  |  |  |
| Power Voltage(50Hz) | V | A:AC (165～250)V ；B： AC(35～60)V |  |
| Power consumption | VA | 8 |  |
| Impulse Resisting Voltage | Kv | ＞5 |  |
| Dimension | Mm | 178(L) x 100 (W) x55(H) |  |

Hangzhou Prevail Communication Technology Co., Ltd

Hangzhou Prevail Optoelectronic Equipment Co., Ltd

Statement：Both  are the registered trademarks of our company. We have the right to use either of them.